LAPORAN RESMI

PRAKTEK PACKET DAN SIRCUIT SWITCHING

Disusun untuk Memenuhi Tugas

Mata Kuliah:

PRAKTEK KONSEP JARINGAN

Oleh:

JUNDIAH INDIRA MILKA TSAURI

3122521020

PROGRAM STUDI D3 TEKNIK INFORMATIKA

PSDKU LAMONGAN

POLITEKNIK ELEKTRONIKA NEGERI SURABAYA

2023

GAMBAR HASIL AKHIR PERCOBAAN:

Langkah-langkah:

- Langkah pertama dari praktek ini, kita membuat desain jaringan di aplikasi Cisco Packet Tracer

Router

Switch

Device

- Menghubungkan komponen-komponen dengan kabel Copper

Kabel straight untuk menyambungkan router ke switch, dan switch ke device

Kabel cross untuk menyambungkan switch ke switch

- Konfigurasi router

Router>enable

Router#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#interface fastEthernet 0/0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0, changed state to up

Router(config-if)#exit

Router(config)#interface fastEthernet 0/0.1

Router(config-subif)#

%LINK-5-CHANGED: Interface FastEthernet0/0.1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0.1, changed state to up

Router(config-subif)#no shutdown

Router(config-subif)#encapsulation dot1Q 10

Router(config-subif)#ip address 192.168.0.1 255.255.255.240

Router(config-subif)#no shutdown

Router(config-subif)#exit

Router(config)#interface fastEthernet 0/0.2

Router(config-subif)#

%LINK-5-CHANGED: Interface FastEthernet0/0.2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0.2, changed state to up

Router(config-subif)#encapsulation dot1Q 20

Router(config-subif)#ip address 192.168.1.1 255.255.255.224

Router(config-subif)#no shutdown

Router(config-subif)#exit

Router(config)#interface fastEthernet 0/0.3

Router(config-subif)#

%LINK-5-CHANGED: Interface FastEthernet0/0.3, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0.3, changed state to up

Router(config-subif)#encapsulation dot1Q 30

Router(config-subif)#ip address 192.168.2.1

% Incomplete command.

Router(config-subif)#ip address 192.168.2.1 255.255.255.128

Router(config-subif)#no shutdown

Router(config-subif)#exit

Router(config)#ip dhcp pool vlan10

Router(dhcp-config)#default-router 192.168.0.1

Router(dhcp-config)#network 192.168.0.0 255.255.255.240

Router(dhcp-config)#exit

Router(config)#ip dhcp pool vlan20

Router(dhcp-config)#default-router 192.168.1.1

Router(dhcp-config)#network 192.168.1.0 255.255.255.224

Router(dhcp-config)#exit

Router(config)#ip dhcp pool vlan30

Router(dhcp-config)#default-router 192.168.2.1

Router(dhcp-config)#network 192.168.2.0 255.255.255.128

Router(dhcp-config)#exit

Router(config)#ip dhcp excluded-address 192.168.0.1 192.168.0.2

Router(config)#ip dhcp excluded-address 192.168.1.1 192.168.1.5

Router(config)#ip dhcp excluded-address 192.168.2.1 192.168.2.10

Router(config)#

Hasil konfigurasi dapat dilihat ketika kursor mengarah ke router:

- Konfigurasi switch0

Switch>enable

Switch#ls

Translating "ls"...domain server (255.255.255.255)

% Unknown command or computer name, or unable to find computer address

Switch#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#vlan 10

Switch(config-vlan)#vlan 20

Switch(config-vlan)#vlan 30

Switch(config-vlan)#exit

Switch(config)#exit

Switch#

%SYS-5-CONFIG_I: Configured from console by console

Switch#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#vlan 10

Switch(config-vlan)#name HRD

Switch(config-vlan)#exit

Switch(config)#vlan 20

Switch(config-vlan)#name Manager

Switch(config-vlan)#exit

Switch(config)#vlan 30

Switch(config-vlan)#

Switch(config-vlan)#name Finance

Switch(config-vlan)#exit

Switch(config)#exit

Switch#

%SYS-5-CONFIG_I: Configured from console by console

Switch#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#interface fastEthernet 0/2

Switch(config-if)#switchport mode trunk

Switch(config-if)#switchport trunk allowed vlan 10-30

Switch(config-if)#exit

Switch(config)#interface fastEthernet 0/4

Switch(config-if)#switchport mode access

Switch(config-if)#switchport access vlan 10

Switch(config-if)#exit

Switch(config)#interface fastEthernet 0/3

Switch(config-if)#switchport mode access

Switch(config-if)#switchport access vlan 10

Switch(config-if)#exit

Switch(config)#interface fastEthernet 0/3

Switch(config-if)#switchport mode access

Switch(config-if)#switchport access vlan 20

Switch(config-if)#exit

Switch(config)#interface fastEthernet 0/5

Switch(config-if)#switchport mode access

Switch(config-if)#switchport access vlan 30

Switch(config-if)#exit

Hasil konfigurasi dapat dilihat ketika kursor mengarah ke Switch0:

- Konfigurasi switch1

Switch>enable

Switch#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#vlan 10

Switch(config-vlan)#vlan 20

Switch(config-vlan)#vlan 30

Switch(config-vlan)#exit

Switch(config)#vlan

% Incomplete command.

Switch(config)#vlan 10

Switch(config-vlan)#name HRD2

Switch(config-vlan)#exit

Switch(config)#vlan 20

Switch(config-vlan)#name Manager2

Switch(config-vlan)#exit

Switch(config)#vlan 30

Switch(config-vlan)#name Finance2

Switch(config-vlan)#exit

Switch(config)#interface fastEthernet 0/1

Switch(config-if)#switchport mode trunk

Switch(config-if)#switchport trunk allowed vlan 10-30

Switch(config-if)#exit

Switch(config)#interface fastEthernet 0/4

Switch(config-if)#switchport mode access

Switch(config-if)#switchport access vlan 10

Switch(config-if)#exit

Switch(config)#interface fastEthernet 0/3

Switch(config-if)#switchport mode access

Switch(config-if)#switchport access vlan 20

Switch(config-if)#exit

Switch(config)#interface fastEthernet 0/5

Switch(config-if)#swicthport mode access

^

% Invalid input detected at '^' marker.

Switch(config-if)#switchport mode access

Switch(config-if)#switchport access vlan 30

Switch(config-if)#exit

Switch(config)#interface fastEthernet 0/2

Switch(config-if)#switchport mode trunk

Switch(config-if)#switchport trunk allowed vlan 10-30

Switch(config-if)#exit

Hasil konfigurasi dapat dilihat ketika kursor mengarah ke Switch1:

- Konfigurasi switch2

Switch>enable

Switch#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#vlan 10

Switch(config-vlan)#vlan 20

Switch(config-vlan)#vlan 30

Switch(config-vlan)#exit

Switch(config)#vlan 10

Switch(config-vlan)#name HRD3

Switch(config-vlan)#exit

Switch(config)#vlan 20

Switch(config-vlan)#name Manager3

Switch(config-vlan)#exit

Switch(config)#vlan 30

Switch(config-vlan)#name Finance3

Switch(config-vlan)#exit

Switch(config)#interface fastEthernet 0/1

Switch(config-if)#switchport mode trunk

Switch(config-if)#switchport trunk allowed vlan 10-30

Switch(config-if)#exit

Switch(config)#interface fastEthernet 0/2

Switch(config-if)#switchport mode access

Switch(config-if)#switchport access vlan 20

Switch(config-if)#exit

Switch(config)#interface fastEthernet 0/3

Switch(config-if)#switchport mode access

Switch(config-if)#switchport access vlan 10

Switch(config-if)#exit

Switch(config)#interface fastEthernet 0/4

Switch(config-if)#switchport mode access

Switch(config-if)#switchport access vlan 30

Switch(config-if)#exit

Switch(config)#

Hasil konfigurasi dapat dilihat ketika kursor mengarah ke Switch2:

- Investigasi vlan dengan perintah show vlan

- Untuk mendapatkan IP Manager klik kursor ke arah PC Manager, kemudian pilih Desktop dan masuk ke IP Configuration

Pada halaman IP Configuration, ubah Static menjadi DHCP:

Begitu pula untuk mendapatkan IP HRD

IP Finance

IP Manager2

IP HRD2

IP Finance2

IP Manager3

IP HRD3

IP Finance3

- Untuk mengecek koneksi HRD ke HRD2, masuk ke halaman Command Prompt

Dan ketik seperti pada gambar berikut:

Cek juga koneksi Manager ke Manager2

Cek koneksi Finance ke Finance2

Cek koneksi HRD2 ke HRD3

Cek koneksi Manager2 ke Manager3

Cek koneksi Finance2 ke Finance3

Jika hasil ping seperti pada gambar, maka artinya sudah berhasil terkoneksi.